# BUILDING MATERIALS

Building material is material used for construction. Many naturally occurring substances, such as clay, rocks, sand, wood, and even twigs and leaves, have been used to construct buildings. Apart from naturally occurring materials, many man-made products are in use, some more and some less synthetic. The manufacturing of building materials is an established industry in many countries and the use of these materials is typically segmented into specific specialty trades, such as carpentry, insulation, plumbing, and roofing work. They provide the make-up of habitats and structures including homes.

Some of the Building Materials

- Sand                       10. Timber

- Wood                     11. Granite

- Glass                       12. Gravel

- Cement

- Steel

- Plastic

- Brick

- Concrete

- Aggregates

# Sand

# Introduction

Sand is a type of naturally occurring material that is of a granular, loose, fragmented composition, consisting of particulate matter such as rock, coral, shells, and so on. Sand is typically finer than gravel but coarser than silt.

The precise composition of sand varies depending on its source and the conditions prevalent at that location. In in-land continental regions, the predominant constituent of sand is silica (silicon dioxide), typically in the form of quartz. Sand that has been created over millions of years by such things as coral and shellfish is typically aragonite, which is a form of calcium carbonate.

# Uses of sand

Sand is very commonly used in construction, often providing bulk, strength, and stability to other materials such as asphalt, concrete, mortar, render, cement, and screed. Sand is also used as a base layer is known as ‘blinding’, which is laid above a layer of hardcore to provide a clean, level, and dry surface for construction works. It can also be used in its raw form as a decorative material in landscaping.

Sand is used in liquid form to manufacture glass and is also used for molding metal casting. It can be used as an abrasive in the process of sandblasting which cleans structural elements, steelwork, and so on. Sandpaper is also made using sand.

## Manufacture

Manufactured sand (M sand) is sand made from rock by artificial processes, usually for construction purposes in cement or concrete. It differs from river sand by being more angular, and has somewhat different properties.

## Resources and environmental concerns

Only some sands are suitable for the construction industry, for example for making concrete. Because of the growth of population and of cities and the consequent construction activity there is a huge demand for these special kinds of sand, and natural sources are running low. In 2012 French director Denis Delestrac made a documentary called "Sand Wars" about the impact of the lack of construction sand. It shows the ecological and economic effects of both legal and illegal trade in construction sand.[18][19][20]

To retrieve the sand, the method of hydraulic dredging is used. This works by pumping the top few meters of sand out of the water and filling it into a boat, which is then transported back to land for processing. Unfortunately, all marine life mixed in with the extracted sand is killed and the ecosystem can continue to suffer for years after the mining is complete. Not only does this affect marine life, but also the local fishing industries because of the loss of life, and communities living close to the water's edge. When sand is taken out of the water it increases the risk of landslides, which can lead to loss of agricultural land and/or damage to dwellings.[21]

Sand's many uses require a significant dredging industry, raising environmental concerns over fish depletion, landslides, and flooding.[22] Countries such as China, Indonesia, Malaysia, and Cambodia ban sand exports, citing these issues as a major factor.[23] It is estimated that the annual consumption of sand and gravel is 40 billion tons and sand is a US$70 billion global industry.[24] With increasing use, more is expected to come from recycling and alternatives to sand.

# Aggregate

# Introduction

Aggregate is the term given to material frequently used in construction as a means of stabilising and reinforcement. Aggregates are widely used in drainage applications and as base material under foundations and roads. In the 10 years ending in 2010, 2.5 billion tonnes of aggregates were consumed by the UK. These aggregates were supplied from various sources:

# The largest source is from quarrying of the land for sand and gravel, or for rock suitable for crushing (mainly limestone, sandstone, and igneous rock, but also some unusually hard occurrences of chalk, ironstone, slate, etc.).

# Sand and gravel is dredged from the seabed and is a regionally significant source

# Uses

As a basic raw material aggregates can be put to many uses, although certain tasks may require a specific type of aggregate. A large proportion of aggregate is used to manufacture concrete, as well as the cement that is used in the concrete.

Aggregates are used in construction to provide drainage, fill voids, protect pipes, and to provide hard surfaces.

Aggregates are not just used for construction. A vast array of other products, many of which are taken for granted, are manufactured using materials derived from quarrying. These include: paper, glass, cosmetics and toothpaste to name but a few. Aggregates are also used in agriculture, food manufacture, and water and sewage purification. Water will percolate through a trench filled with aggregate more quickly than it will through the surrounding soil, thus enabling an area to be drained of surface water. This is frequently used alongside roads in order to disperse water collected from the asphalt surfacing.

Voids created around the foundations of buildings during construction are filled with aggregate because it is easier to compact than the original soil that was removed, resulting in a more solid finish that will support the structure. Aggregates generally are not affected by the weather as much as soils, particularly clay soils, and will not suffer from shrinkage cracking during dry spells.

Pipes laid to convey treated water, or as conduits for cables, need to be protected from sharp objects in the ground and are therefore laid on, and surrounded by, fine aggregate before trenches are backfilled.

Unpaved roads and parking areas are covered in a surface layer of aggregate to provide a more solid surface for vehicles, from bicycles to lorries. This prevents the vehicles from sinking into the soil, particularly during wet weather.

Concrete

# Introduction

Concrete is the most commonly used man-made material on earth. It is an important construction material used extensively in buildings, bridges, roads and dams. Its uses range from structural applications, to paviours, kerbs, pipes and drains.

Concrete is a composite material, consisting mainly of Portland cement, water and aggregate (gravel, sand or rock). When these materials are mixed together, they form a workable paste which then gradually hardens over time.

# Benefits of concrete

There are numerous positive aspects of concrete:

- It is a relatively cheap material and has a relatively long life with few maintenance requirements.

- It is strong in compression.

- Before it hardens it is a very pliable substance that can easily be shaped.

- It is non-combustible.

# Characteristics of concrete

The characteristics of concrete are determined by the aggregate or cement used, or by the method that is used to produce it. The water-to-cement ratio is the determining factor in ordinary structural concrete with a lower water content resulting in a stronger concrete.

This, however, reduces the workability (and pumpability) of the concrete, which can be measured using the slump test. The grading, shape, texture and proportion of aggregate can also have a similar affect. If a particularly strong concrete is required, the amount of aggregate can be reduced in relation to the cement. However, cement is a significant cost factor, and increasing its proportion in the mix will increase the overall price.

# Cement

# Introduction

Cement is a substance used for binding and hardening other materials. Water and cement set and harden through a chemical reaction known as 'hydration'. The process of hardening is described as 'curing', which requires particular conditions of temperature and humidity.

Cement can be mixed with a fine aggregate and water to produce mortar, used in masonry construction as a bedding and adhesive to bind and fill the gaps between adjacent blocks of brick, concrete or stone.

It can be also be mixed with water, aggregates (such as gravel, sand or rock), and sometimes admixtures, to form concrete, and can be used to make renders, screeds and so on. The ratio of water and cement will determine the overall strength and quality of the mix.

The exact properties of the cement paste are very important:

- It must be fluid enough for some time after mixing to allow the mix to be formed into its final shape.

- It must then set and gain strength so that it binds the aggregates together to produce a strong material.

# Types of cement

## Portland cement

## Low heat blast-furnace Portland cement

## Rapid hardening cement

## Sulphate resisting cement

## High alumina cement

## Ferro-cement

# Glass

# Introduction

Glass is a material made from liquid sand. It is the name given to any amorphous (non-crystalline) solid that displays a glass transition near its melting point which is around 1,700°C (3,090°F). This means that materials transform from a hard and brittle state into a molten state, or vice versa depending on whether the glass transition temperature is the melting or solidifying point. An amorphous solid has some of the crystalline order of a solid and some of the random molecular structure of a liquid.

Silicate glass is the most common form, which consists mainly of silica or silicon dioxide, SiO2. Impurities or additional elements and compounds added to the silicate to change the color and other properties of the glass.

Glass is a very commonly used material because, whilst still molten, it can be manipulated into forms suitable for a very wide range of different uses, from packaging and household objects to car windscreens, windows, and so on.

# Properties of glass

The composition of glass and the cooling rate can be varied to give a range of properties depending on the end use required:

- Visual transparency and reflectance can be altered with the addition of admixtures to the initial batch mix. Glass can be transparent, translucent, tinted, reflective, stained, opaque, and so on.

- Solar optical properties can be controlled to transmit, absorb or reflect specific wavelengths of the solar spectrum.

- Long wave infra-red optical properties can be varied to affect emissivity, for example to create low-e glass.

- U value can be altered by the R-value of layers of glass and their surface heat transfers.

- Strength can be altered with laminates and admixtures that increase the ability of glass to resist deformation, shearing or shattering under load.

- Glass can be worked in many different ways, from blowing, drawing and pressing to welding.

- Glass is 100% recyclable. Scraps of waste glass are used as raw materials in glass manufacture and as aggregates in concrete manufacture. The number of times glass has been recycled does not affect its quality, strength or functionality

Wood

Wood is one of the most used natural building materials in the world. A number of valuable properties such as low heat conductivity, small bulk density, relatively high strength, amenability to mechanical working etc. makes wood as famous building material. The use of wood can reduce construction costs by up to 30%, with a low index of material waste, and speed up construction, spending half the time used in masonry construction. The wood can be applied in finishings, as concrete forms and as structural pieces, from lumber, squares and logs.

Performance Properties

Wood performance is dependent on a wide range of characteristics, the importance of which depends on how the piece is intended to be used.

- Strength

- stiffness

- Hardness

- Finish

- Retention

- Treatability with Preservatives

- Resistance to shrinkage, swelling, warping, checking weathering decay photodegradation color changes insect attack

## Uses of Wood in Different Sectors

Wood is a plant part having multipurpose uses those are impossible to deny and difficult to note all in our daily life. From the ancient time wood is used by human and this continuation still remains in the modern civilization. A few of many uses of wood are mentioned below:

### Construction and Fencing                       Sports Equipment

### Household Uses                                      Art Industry

### Commercial Uses

# Steel

Steel is an alloy made up of iron with typically a few tenths of a percent of carbon to improve its strength and fracture resistance compared to other forms of iron. Many other elements may be present or added. Stainless steels that are corrosion- and oxidation-resistant need typically an additional 11% chromium. Because of its high tensile strength and low cost, steel is used in buildings, infrastructure, tools, ships, trains, cars, machines, electrical appliances, and weapons. Iron is the base metal of steel. Depending on the temperature, it can take two crystalline forms (allotropic forms): body-centred cubic and face-centred cubic. The interaction of the allotropes of iron with the alloying elements, primarily carbon, gives steel and cast iron their range of unique properties.

In pure iron, the crystal structure has relatively little resistance to the iron atoms slipping past one another, and so pure iron is quite ductile, or soft and easily formed. In steel, small amounts of carbon, other elements, and inclusions within the iron act as hardening agents that prevent the movement of dislocations.

The carbon in typical steel alloys may contribute up to 2.14% of its weight.[1] Varying the amount of carbon and many other alloying elements, as well as controlling their chemical and physical makeup in the final steel (either as solute elements, or as precipitated phases), impedes the movement of the dislocations that make pure iron ductile, and thus controls and enhances its qualities. These qualities include the hardness, quenching behaviour, need for annealing, tempering behaviour, yield strength, and tensile strength of the resulting steel. The increase in steel's strength compared to pure iron is possible only by reducing iron's ductility.

### Properties[edit]

The density of steel varies based on the alloying constituents but usually ranges between 7,750 and 8,050 kg/m3 (484 and 503 lb/cu ft), or 7.75 and 8.05 g/cm3 (4.48 and 4.65 oz/cu in).[7]

Even in a narrow range of concentrations of mixtures of carbon and iron that make steel, several different metallurgical structures, with very different properties can form. Understanding such properties is essential to making quality steel. At room temperature, the most stable form of pure iron is the body-centred cubic (BCC) structure called alpha iron or α-iron. It is a fairly soft metal that can dissolve only a small concentration of carbon, no more than 0.005% at 0 °C (32 °F) and 0.021 wt% at 723 °C (1,333 °F). The inclusion of carbon in alpha iron is called ferrite. At 910 °C, pure iron transforms into a face-centred cubic (FCC) structure, called gamma iron or γ-iron. The inclusion of carbon in gamma iron is called austenite. The more open FCC structure of austenite can dissolve considerably more carbon, as much as 2.1%[8] (38 times that of ferrite) carbon at 1,148 °C (2,098 °F), which reflects the upper carbon content of steel, beyond which is cast iron.[9] When carbon moves out of solution with iron, it forms a very hard, but brittle material called cementite (Fe3C).

### Uses

### Long steel

- As reinforcing bars and mesh in reinforced concrete

- Railroad tracks

- Structural steel in modern buildings and bridges

- Wires

- Input to reforging applications

### Flat carbon steel[

- Major appliances

- Magnetic cores

- The inside and outside body of automobiles, trains, and ships.

### Weathering steel (COR-TEN)

- Intermodal containers

- Outdoor sculptures

- Architecture

- Highliner train cars

### Stainless steel

- Cutlery

- Rulers

- Surgical instruments

- Watches

- Guns

- Rail passenger vehicles

- Tablets

- Trash Can

- Body piercing jewellery

- Inexpensive rings

- Components of spacecraft and space stations

### Low-background steel

Steel manufactured after World War II became contaminated with radionuclides by nuclear weapons testing. Low-background steel, steel manufactured prior to 1945, is used for certain radiation-sensitive applications such as Geiger counters and radiation shielding.

# Plastic

### Plastics are a wide range of synthetic or semi-synthetic materials that use polymers as a main ingredient. Their plasticity makes it possible for plastics to be moulded, extruded or pressed into solid objects of various shapes. This adaptability, plus a wide range of other properties, such as being lightweight, durable, flexible, and inexpensive to produce, has led to its widespread use. Plastics typically are made through human industrial systems. Most modern plastics are derived from fossil fuel-based chemicals like natural gas or petroleum; however, recent industrial methods use variants made from renewable materials, such as corn or cotton derivatives.

## Merits

- Plastics are strong yet lightweight, and so they are easy to transport & manoeuvre.

- They are durable, knock-and scratch resistant with excellent weatherability.

- They do not rot or corrode.

- Plastics are easy to install; many have a snap-fit kind of jointing procedures.

- Plastics offer limitless possibilities in design achieved by extrusion, bending, moulding etc.

- They can be given any range of colours by adding pigments.

- The plastics are low conductors of heat and thus are used as insulation materials in green building concepts.

- The plastics products can achieve tight seals.

- Plastic doesn't break easily

- They can be sawn and nailed employing standard carpentry tools and skills.

- They can be easily removed and recycled.

- They are poor conductors of electricity.

## Disadvantages and limitations

- Plastics may be degraded under the action of direct sunlight which reduces their mechanical strength.

- Many plastics are flammable unless treated.

- High embodied energy content

- Low modulus of elasticity: makes them unsuitable for load-bearing applications.

- Thermoplastics are subject to creep and soften at moderate temperatures.

- Thermal expansion for most plastics is high: adequate thermal movement has to be allowed in detailing.

- Many types of plastics are not biodegradable thus cause pollution when they accumulate.

## Products

Some of the examples below are Products of Plastics in the Construction industry:

- Pipes  : Electrical Conduits, Rain Water & Sewage pipes, Plumbing, Gas Distributions.

- Cables : PVC Insulation on cables, Insulation Tapes .

- Floorings : Flooring tiles & Rolls .

- Domes / sky lights : Opaque as well as transparent.

- Roofing  : Coloured or Double skinned for insulation.

- Windows & doors : Extruded sections for Door and windows and panels.

- Storage tanks : Storage tanks.

- Hardware accessories : Washers, Nut bolts, Sleeves, Anchoring wires.

# Brick

A brick is a type of block used to build walls, pavements and other elements in masonry construction. Properly, the term brick denotes a block composed of dried clay, but is now also used informally to denote other chemically cured construction blocks. Bricks can be joined using mortar, adhesives or by interlocking them.[1][2] Bricks are produced in numerous classes, types, materials, and sizes which vary with region and time period, and are produced in bulk quantities.

Block is a similar term referring to a rectangular building unit composed of similar materials, but is usually larger than a brick. Lightweight bricks (also called lightweight blocks) are made from expanded clay aggregate.

Fired bricks are one of the longest-lasting and strongest building materials, sometimes referred to as artificial stone, and have been used since circa 4000 BC. Air-dried bricks, also known as mudbricks, have a history older than fired bricks, and have an additional ingredient of a mechanical binder such as straw.

Bricks are laid in courses and numerous patterns known as bonds, collectively known as brickwork, and may be laid in various kinds of mortar to hold the bricks together to make a durable structure.

## Uses

In the United States, bricks have been used for both buildings and pavements. Examples of brick use in buildings can be seen in colonial era buildings and other notable structures around the country. Bricks have been used in pavements especially during the late 19th century and early 20th century. The introduction of asphalt and concrete reduced the use of brick pavements, but they are still sometimes installed as a method of traffic calming or as a decorative surface in pedestrian precincts. For example, in the early 1900s, most of the streets in the city of Grand Rapids, Michigan, were paved with bricks. Today, there are only about 20 blocks of brick-paved streets remaining (totalling less than 0.5 percent of all the streets in the city limits).[33] Much like in Grand Rapids, municipalities across the United States began replacing brick streets with inexpensive asphalt concrete by the mid-20th century.[34]

Bricks in the metallurgy and glass industries are often used for lining furnaces, in particular refractory bricks such as silica, magnesia, chamotte and neutral (chromomagnesite) refractory bricks. This type of brick must have good thermal shock resistance, refractoriness under load, high melting point, and satisfactory porosity. There is a large refractory brick industry, especially in the United Kingdom, Japan, the United States, Belgium and the Netherlands.

In Northwest Europe, bricks have been used in construction for centuries. Until recently, almost all houses were built almost entirely from bricks. Although many houses are now built using a mixture of concrete blocks and other materials, many houses are skinned with a layer of bricks on the outside for aesthetic appeal.

Engineering bricks are used where strength, low water porosity or acid (flue gas) resistance are needed.

In the UK a red brick university is one founded in the late 19th or early 20th century. The term is used to refer to such institutions collectively to distinguish them from the older Oxbridge institutions, and refers to the use of bricks, as opposed to stone, in their buildings.

# Granite

Granite  is a coarse-grained (phaneritic) intrusive igneous rock composed mostly of quartz, alkali feldspar, and plagioclase. It forms from magma with a high content of silica and alkali metal oxides that slowly cools and solidifies underground. It is common in the continental crust of Earth, where it is found in igneous intrusions. These range in size from dikes only a few centimeters across to batholiths exposed over hundreds of square kilometers.

Granite is typical of a larger family of granitic rocks, or granitoids, that are composed mostly of coarse-grained quartz and feldspars in varying proportions. These rocks are classified by the relative percentages of quartz, alkali feldspar, and plagioclase (the QAPF classification), with true granite representing granitic rocks rich in quartz and alkali feldspar. Most granitic rocks also contain mica or amphibole minerals, though a few (known as leucogranites) contain almost no dark minerals.

Granite is nearly always massive (lacking any internal structures), hard, and tough. These properties have made granite a widespread construction stone throughout human history.

### Physical properties[edit]

The average density of granite is between 2.65 and 2.75 g/cm3 (165 and 172 lb/cu ft),[10] its compressive strength usually lies above 200 MPa, and its viscosity near STP is 3–6·1020 Pa·s.[11]

The melting temperature of dry granite at ambient pressure is 1215–1260 °C (2219–2300 °F);[12] it is strongly reduced in the presence of water, down to 650 °C at a few hundred megapascals of pressure.[13]

Granite has poor primary permeability overall, but strong secondary permeability through cracks and fractures if they are present.

### Chemical composition[edit]

A worldwide average of the chemical composition of granite, by weight percent, based on 2485 analyses:[14]

| SiO2 | 72.04% (silica) | TiO2 | 0.30% |
| --- | --- | --- | --- |
| Al2O3 | 14.42% (alumina) | P2O5 | 0.12% |
| K2O | 4.12% | MnO | 0.05% |
| Na2O | 3.69% | Fe2O3 |  |  | 1.22% |
| CaO | 1.82% | MgO |  |  | 0.71% |
| FeO | 1.68% |
| GRAVEL |  |

Gravel  is a loose aggregation of rock fragments. Gravel occurs naturally throughout the world as a result of sedimentary and erosive geologic processes; it is also produced in large quantities commercially as crushed stone.

Gravel is classified by particle size range and includes size classes from granule- to boulder-sized fragments. In the Udden-Wentworth scale gravel is categorized into granular gravel (2–4 mm or 0.079–0.157 in) and pebble gravel (4–64 mm or 0.2–2.5 in). ISO 14688 grades gravels as fine, medium, and coarse, with ranges 2–6.3 mm to 20–63 mm. One cubic metre of gravel typically weighs about 1,800 kg (or a cubic yard weighs about 3,000 lb).

Gravel is an important commercial product, with a number of applications. Almost half of all gravel production is used as aggregate for concrete. Much of the rest is used for road construction, either in the road base or as the road surface (with or without asphalt or other binders.) Naturally occurring porous gravel deposits have a high hydraulic conductivity, making them important aquifers.

## Production and uses

Gravel is a major basic raw material in construction.[24] Sand is not usually distinguished from gravel in official statistics, but crushed stone is treated as a separate category. In 2020, sand and gravel together made up 23% of all industrial mineral production in the U.S., with a total value of about $12.6 billion. Some 960 million tons of construction sand and gravel were produced. This greatly exceeds production of industrial sand and gravel (68 million tons), which is mostly sand rather than gravel.[25]

It is estimated that almost half of construction sand and gravel is used as aggregate for concrete. Other important uses include in road construction, as road base or in blacktop; as construction fill; and in myriad minor uses.[24]

Gravel is widely and plentifully distributed, mostly as river deposits, river flood plains, and glacial deposits, so that environmental considerations and quality dictate whether alternatives, such as crushed stone, are more economical. Crushed stone is already displacing natural gravel in the eastern United States, and recycled gravel is also becoming increasingly important.

Types of gravel include:

- Bank gravel: naturally deposited gravel intermixed with sand or clay found in and next to rivers and streams. Also known as "bank run" or "river run".[28]

- Bench gravel: a bed of gravel located on the side of a valley above the present stream bottom, indicating the former location of the stream bed when it was at a higher level. The term is most commonly used in Alaska and the Yukon Territory.[29]

- Crushed stone: rock crushed and graded by screens and then mixed to a blend of stones and fines. It is widely used as a surfacing for roads and driveways, sometimes with tar applied over it. Crushed stone may be made from granite, limestone, dolomite, and other rocks. Also known as "crusher run", DGA (dense grade aggregate) QP (quarry process), and shoulder stone.[30] Crushed stone is distinguished from gravel by the U.S. Geological Survey.[25]

- Fine gravel: gravel consisting of particles with a diameter of 2 to 6.3 millimetres (0.079 to 0.248 in)[13]

- Stone dust: fine, crushed, gravel from the final stage of screen separation, such that the gravel is not separated out from fine dust particles. As with other forms of crushed stone, this is distinguished from gravel by the U.S. Geological Survey.[25]

- Lag gravel: a surface accumulation of coarse gravel produced by the removal of finer particles.

- Pay gravel: also known as "pay dirt"; a nickname for gravel with a high concentration of gold and other precious metals. The metals are recovered through gold panning.

- Pea gravel: also known as "pea shingle" is clean gravel similar in size to garden peas.[31] Used for concrete surfaces, walkways, driveways and as a substrate in home aquariums.

- Piedmont gravel: a coarse gravel carried down from high places by mountain streams and deposited on relatively flat ground, where the water runs more slowly.[32]

- Plateau gravel: a layer of gravel on a plateau or other region above the height at which stream-terrace gravel is usually found.

Timber

Timber is one of the most useful and important material for constructions. Selecting timber is not an easy task, because timber has different types out of which selecting the right material is an important key. Timber is an expensive material to be incorporated in a building for different purpose therefore it should necessarily be strong, tough and durable. Timber doors or windows and etc. contribute a lot in the beautification and overall look of interiors. Timber is used in doors, windows, cabinet, cupboards, shelves, tables and railings etc. Timber is also popularly used in the form of plywood & raw wood. Products like ply blocks and ply boards. Heavy patterned doors and windows are made of solid wood/Timber to provide the strength, toughness and durability.

Type of timber to be used for right purpose is important because if timber used in construction is of low quality then this may need replacement. While selecting timber one should consider its quality aspect as timber must be free from decay like rotten, fungi and termite. Following are some essential tips and guidelines to know about timber selection for buildings:

- Teakwood is a most appropriate timber to be used in the construction of buildings. Teakwood is naturally durable with good merit and adaptability although it is prone to attacks of white ants and insects. Yet builders choose teakwood for its natural look, durability with long lasting characteristics.

- Sal is another timber material identified as much stronger and quite harder than teakwood used in building purposes. Sal is less prone to termite or fungi attack and can be used for several constructive purposes.

- Deodar is one of a strongest Indian conifer with less strength and weight than teakwood. Deodar has maintained its good image for durability and style of appearance. Deodar can be easily cast in any shape, easy to saw. Generally used for building & construction in homes and other civil structures.

- Hardwood is another form of wood which is treated thoroughly before use as it is not durable as much as teakwood or deodar. After treated through, wood preservative and polishing on hardwood makes it suitable for house building structure.

- Ply board is used in buildings for various purposes. They are formed by pressing together several layers of wood.

- Particle board is made of agro waste, cellulose etc and blended with adhesive to make into a solid board.

# Elements of Civil Engineering

# Assignment – 1

NAME : VAIBHAV C

SR.NO : 21-1120

BRANCH & SEC : AIML (P-2)

TOPIC : BUILDING MATERIALS